EEE373 – Digital System Design

Department of Electrical Engineering & Electronics

Assignment 3_Report

-MIPS Processor Design-

| Student Name: | Yu-Cheng. Lin |
| --- | --- |
| Student ID: | 201447973 |

Abstract

Assignment 3 is about MIP processor design, divided into part A and part B. The Part A is to modify the MIPS assembly language program so that the program displays the lowest 8 digits of the student ID on the 7-segment display of the DE2 board. The Part B requires that the design of MIPS be modified, and specified new instructions should be added when a limited number of MIPS instructions can be used normally. The purpose of completing Assignment 3 is to understand the internal operation of the MIPS processor, learn how to use the ASM design method to design a digital system and use Verilog hardware to implement a digital system description language.

## Table of Contents

Abstract

Table of Contents

1	Part_A	1

2	Part_B	3

3	Discussion	6

Appendix	7

1. Part_A

The ID used in Part A is 201447973 and the lowest 8 digits of it are 01447973. These 8 digits need to be changed into corresponding hexadecimal codes 40, 79, 19, 19, 78, 10, 78, 30 in order to be displayed on the 7-segment display of the DE2 board. Therefore, on the existing MIPS system, the MIPS assembly language program is modified in ‘insts_data’ as shown in Figure 1.

Figure 1: Encoding used in Part A (Code)

The encoding of Figure 1 is obtained by decoding the MIPS assembly code of Figure 2. Since the lui instruction can only generate the first 16 bits, use the addi instruction to add the lowest 16 bits. The $3 register is used to determine the HEX0 to HEX7 of the DE2 board. Finally, the sw instruction is used to store the hexadecimal code to be displayed into the $3 register. Note that the $3 register changes the stored HEX through +4, where 0xFFFF_2010 is HEX0.

Figure 2: MIPS Assembly Code (Source)

Figure 3 is the result of the simulation of the MIPS system through ModelSim.

Figure 3: The simulation results of Part A

When KEY is ‘low’, the system is activated. The first one is that the output of HEX0 is 30 in hexadecimal and 3 is displayed on the DE2 board.

2. Part_B

In Part B, the design of MIPS is modified to add nor, xori and lb instructions. Mainly designed according to the block diagram in Figure 4.

Figure 4: The Block Diagram of Part B

The code is in the appendix. The ASM chart is drawn through the code as shown in Figure 5. Each instruction has its own code. According to the input code, the MIPS system can determine the instruction used. However, lb, xori and nor are different types of instructions, thereby op and funct are used to distinguish between them. The lb and xori are I-type instructions. The nor is R-type instruction.

Figure 5: The ASM of Part B

In order to test the performance of the instruction, the code in Figure 6 was entered into the ‘insts_data’.

Figure 6: The encoding of Part B

The results of the test are shown in Figure 7. The result of nor instruction will be ‘1’ only when a=’0’ and b=’0’. The result of the xori instruction is ‘1’ only when a=’1’ and b=’0’ or a=’0’ and b=’1’. The lb instruction only accesses the lowest 8 bits to the register, and the remaining 24 bits are expanded according to the 7-bit sign

Figure 7: The simulation results of Part B

It can be seen that by using the nor instruction on 0xff00_ff00 and 0xffff_0000, the result is 0x0000_00ff. Similarly, by using the xori instruction for 0xff00_ff00 and 0x0000_f0f0, the result is 0xff00_0ff0. However, the figure does not see the result of the lb instruction on 0x1234_1234. Therefore, rt is added for simulation, as shown in Figure 8.

Figure 8: The simulation results of rt

According to the code, the obtained result of lb instruction is placed in register 8 and register 9. The results are 0xzzzz_zz34.

3. Discussion

The tasks are all completed, the only problem should be the lb command. The tasks are all completed, the only problem should be the lb command. The code to extend the 7-bit sign is missing. The normal lb instruction will load the lowest 8-bit code, and the remaining 24 bits are determined according to the eighth bit. If it is ‘1’, the others are all ‘1’, and vice versa, if it is ‘0’, the others are all ‘0’. In the test, the lowest 8 bits are ‘0011_0100’, and the result should be 0x0000_0034.

Appendix